3.2	The rules of the Syldavian electricity market stipulate that all participants must trade energy exclusively through the Power Pool. However, the Syldavia Aluminum Company (SALCo) and the Northern Syldavia Power Company (NSPCo) have signed a contract for difference for the delivery of 200 MW on a continuous basis at a strike price of 16 $/MWh.

a. Trace the flow of power and money between these companies when the pool price takes the following values: 16 $/MWh, 18 $/MWh and 13 $/MWh.

The flow of power goes from NSPCo  Pool  SALCo. They agree through a contract for difference that SALCo will buy power from the pool at $16/MWh. If the price for power is lower than SALCo pays the difference so that pricing equates to $16/MWh. If the price of power goes over the agreed price then NSPCo “Reimbursed” SALCo the difference so they don’t overpay.

| Pool price | Pool pays NSPCo | SALCo pays Pool | CfD settlement | NSPCo’s net revenue | SALCo’s net cost |
| --- | --- | --- | --- | --- | --- |
| $16/MWh | $3,200 | $3,200 | No payment | $3,200 | $3,200 |
| $18/MWh | $3,600 | $3,600 | NSPCo pays SALCo $400 | $3,200 | $3,200 |
| $13/MWh | $2,600 | $2,600 | SALCo pays NSPCo $600 | $3,200 | $3,200 |

b. What happens if during one hour the NSPCo is able to deliver only 50 MWh and the pool price is 18 $/MWh?

NSPCo supplies whatever power it can and NSPCo still covers the contract and pays the difference. NSPCo still gets the power from other companies in the pool.

c. What happens if during one hour the SALCo consumes only 100 MWh and the pool price is 13 $/MWh?

SALCo pays the difference to NSPCo, NSPCo still generates the 200 but it goes to the pool for other companies

3.4 The operator of a centralized market for electrical energy has received the bids shown in the table below for the supply of electrical energy during a given period.

| Company | Amount (MWh) | Price($/MWh) |
| --- | --- | --- |
| Red | 100 | 12.5 |
| Red | 100 | 14 |
| Red | 50 | 18 |
| Blue | 200 | 10.5 |
| Blue | 200 | 13 |
| Blue | 100 | 15 |
| Green | 50 | 13.5 |
| Green | 50 | 14.5 |
| Green | 50 | 15.5 |

a. Build the supply curve

b. Assume that this market operates unilaterally, that is, that the demand does not bid and is represented by a forecast. Calculate the market price, the quantity produced by each company and the revenue of each company for each of the following loads: 400 MW, 600 MW, 875 MW.

c. Suppose that instead of being treated as constant, the load is represented by its inverse demand curve, which is assumed to have the following form:

D = L − 4.0 · π

where D is the demand, L is the forecasted load and π is the price. Calculate the effect that this price sensitivity of demand has on the market price and the quantity traded.